# Verification Results Summary

Outcomes of Test Protocols 1 and 2 — Home Audio System

| Project | EE22005 Home Audio System (HAS) — Project Week 4 |
| --- | --- |
| Group | Group HAS (BTA26, FS881, AC3847) |
| Document ID | 20260501_HAS_VerificationResults_v1.docx |
| Version | v2.0 |
| Prepared for | Feasibility Study — Synthesis Evidence (Verification results) |
| Related protocols | PW4_HAS_VER_001 (Equalizer) and PW4_HAS_VER_002 (Class D) |

| Version | Date | Author | Description of changes |
| --- | --- | --- | --- |
| v2.0 | 01/05/2026 | Group HAS (BTA26, FS881, AC3847) | Initial issue compiling results of Test Protocols 1 and 2, with explanations and conclusion. |

## Purpose

This document compiles the measured outcomes of the two formal verification test protocols defined for the Home Audio System: PW4_HAS_VER_001 covering the graphic equalizer and PW4_HAS_VER_002 covering the Class D amplifier. Each result is presented against its acceptance criterion, with a short explanation of any deviation. A combined conclusion follows in Section 5.

## Test Protocol 1 — Equalizer Results

Verification was carried out on Day 2 of the project week using the function generator and four-channel oscilloscope. All measurements were captured as PNG screenshots and stored in the QMS Quality_Control folder per the naming convention.

| Check item | Acceptance criterion | Measured | Pass / Fail | Notes |
| --- | --- | --- | --- | --- |
| BPF1 (bass) centre frequency | 100 Hz ±15 % | 108 Hz | Pass | Within +8 % of nominal — well |

|  |  |  |  | inside ±15 % spec. |
| --- | --- | --- | --- | --- |
| BPF1 peak gain | 12 dB ±1 dB | 11.6 dB | Pass | 0.4 dB low; attributable to E12 rounding of feedback resistor. |
| BPF2 (mid) centre frequency | 1 kHz ±15 % | 0.94 kHz | Pass | −6 % from nominal. |
| BPF2 peak gain | 12 dB ±1 dB | 12.2 dB | Pass | Within ±1 dB tolerance. |
| BPF3 (treble) centre frequency | 8 kHz ±15 % | 7.4 kHz | Pass | −7.5 %; likely op-amp GBW-limited at 8 kHz. |
| BPF3 peak gain | 12 dB ±1 dB | 11.1 dB | Pass | Marginal pass; revisit if increased boost is required. |
| Full-EQ neutral ripple | < 4 dB | 2.6 dB | Pass | Smooth summed response across audio band. |
| Full-EQ full-boost peaks | Three peaks +12 dB ±1 dB | All three present | Pass | Three distinct peaks visible at expected centres. |
| Independent-control check | Adjacent change < 2 dB | 1.4 dB max | Pass | Low cross-coupling between bands. |

### Explanation

All nine equalizer checks passed first time. The slight downward shift on BPF3 (−7.5 % from 8 kHz) is consistent with the gain–bandwidth limitation of the TL072 at that gain setting, and remains comfortably inside the ±15 % tolerance specified by S1. The two gain readings that fell below 12 dB nominal (11.6 dB and 11.1 dB) are within the ±1 dB specification window; they reflect the deliberate decision to round design values to E12 standard components, an outcome already accepted in the choice matrix. Cross-coupling between bands was measured at a maximum of 1.4 dB at adjacent-band centres, well within the 2 dB threshold for usability specification S4.

## Test Protocol 2 — Class D Results

Pre-power and powered verification was carried out on Day 3 (pre-power and gate-drive checks) and Day 4 (RLC filter and DC offset). The earphone hold-point was held closed until Day 5 morning.

| Check item | Acceptance criterion | Measured | Pass / Fail | Notes |
| --- | --- | --- | --- | --- |
| +V rail continuity | < 1 Ω | 0.4 Ω | Pass | Stable; clean joints. |
| –V rail continuity | < 1 Ω | 0.5 Ω | Pass | Within tolerance. |

| Analogue ground continuity | < 1 Ω | 0.3 Ω | Pass | Solid ground return. |
| --- | --- | --- | --- | --- |
| Half-bridge midpoint to RLC input | < 1 Ω | 0.6 Ω | Pass | Acceptable. |
| +V rail to ground (insulation) | > 1 MΩ | 3.1 MΩ | Pass | No short observed. |
| –V rail to ground (insulation) | > 1 MΩ | 2.7 MΩ | Pass | No short observed. |
| Triangle wave frequency | 100 kHz ±5 % | 98.4 kHz | Pass | −1.6 % of nominal. |
| Triangle wave amplitude | Per design ±10 % | 5.8 V pp | Pass | Within tolerance. |
| PWM duty cycle modulation | Visible tracking | Tracking confirmed | Pass | Two-channel capture saved to Quality_Control. |
| Dead time — rising edge | ≥ 0.5 µs | 0.7 µs | Pass | Initially 0.2 µs — failed. |
| Dead time — falling edge | ≥ 0.5 µs | 0.6 µs | Pass | After RC rework. |
| Shoot-through observed | None | None after rework | Pass | Initial RC value too small; reworked, then re-verified. |
| DC offset — no signal | < 0.4 V | 0.12 V | Pass | Comfortable margin. |
| DC offset — low signal (100 mV) | < 0.4 V | 0.18 V | Pass | Within margin. |
| DC offset — full signal | < 0.4 V | 0.31 V | Pass | Closer to limit but within spec. |

### Explanation

The pre-power phase passed without any rework. The first attempt at the dead-time verification (Section 6.6 of Protocol 2) failed: rising-edge dead time measured at approximately 0.2 µs, with a brief overlap visible on the gate-drive capture. This matched the qualitative description of shoot-through risk and the rework instruction in the protocol. The dead-time RC values were increased per the schematic note (RC time constant raised from the original design value), the section was repeated in full, and dead time on both transitions was confirmed at 0.7 µs and 0.6 µs respectively — comfortably above the 0.5 µs threshold and with no overlap observed. This single failure-and-rework cycle is recorded here as evidence that the verification protocol caught a real

defect that would otherwise have caused destructive shoot-through current under sustained operation.

The DC-offset hold-point was released only after all three signal conditions were measured below

0.4 V and signed off by the test lead (FS881). The full-signal case at 0.31 V is the tightest margin in the build and was re-checked after audio playback; it remained stable, supporting safe earphone connection.

## Compliance summary against target specifications

| Spec | Description | Verification source | Outcome |
| --- | --- | --- | --- |
| S1 | Equalizer frequency response (centres, gain, ripple) | Test Protocol 1, sections 6.1–6.5 | Pass |
| S2 | Class D switching efficiency and PWM correctness | Test Protocol 2, sections 6.4–6.6 | Pass (after rework) |
| S3 | RLC output filter ripple and offset | Test Protocol 2, section 6.7 | Pass |
| S4 | Independent per-band control | Test Protocol 1, section 6.6 | Pass |
| S6 | Hearing-safety output limits (DC offset) | Test Protocol 2, section 6.7 (hold-point) | Pass |

## Conclusion

All nine equalizer checks and all fifteen Class D checks ultimately passed against their acceptance criteria, satisfying target specifications S1, S2, S3, S4 and S6. The verification process caught one real defect: insufficient gate-drive dead time, and the documented rework path returned the system to compliance before the earphone-connection hold-point was released. The Home Audio System is therefore judged fit for full integration and final demonstration. Evidence files (oscilloscope screenshots, photographs and recorded measurements) are stored in the QMS Quality_Control folder under the file naming convention defined in.